Auto Feeder Assembly Guide

A step-by-step manual for assembling the single and dual-channel feeders

1. Introduction

Precise and consistent feeding regimens are crucial in biological research—particularly for fast-living model organisms such as the African turquoise killifish—to ensure reliable experimental outcomes. Automating the feeding process mitigates human error, maintains uniform food delivery, and enables researchers to focus on designing, executing, and analysing experiments at greater scale. By facilitating controlled feeding schedules and reducing manual labour, automated feeders substantially accelerate research progress and support larger sample sizes without compromising data quality.

This document details a newly designed automated feeder system that overcomes several limitations of a prior in-lab prototype. The updated system:

- Features a fully enclosed casing that protects sensitive electronic components from the moist environment typical of aquatic research facilities.

- Operates on onboard batteries, eliminating the need for external wiring prone to tangling or wear around the densely packed, humid environment

- Leverages an Arduino microcontroller (notable for extensive documentation and community support) and incorporates a convenient LCD interface to display operational modes and device status.

The purpose of this guide is to provide thorough assembly instructions so that a broad range of users, including those with limited technical experience, can construct and maintain the feeder. By implementing this enclosed, battery-powered feeding system, researchers can ensure reproducible feeding protocols that enable higher throughput and more reliable experimental outcomes, ultimately advancing killifish husbandry and broader scientific endeavours.

2. Materials and Tools Required

This section provides a checklist of all electronic parts, wiring, and general tools needed for assembly. Verify that you have everything on this list before beginning construction.

2.1  Electronic Components:

·   	ESP8266 ESP-01 WiFi Module

·   	RTC Module (1307)

·   	OLED Screen (SSD1306, 128x32)

·   	SG90 Servo Motor(s) (1 for single-feeder, 2 for dual-feeder)

·   	Push Buttons (for manual feed control)

·   	LED and 200-ohm Resistor (1 for single-feeder, 2 for dual-feeder)

·   	Li-ion 18650 (3000 mAh) Batteries (x2) corresponding Battery Holder

·   	Arduino nano

·   	Screw Terminal Adapter (Nano Dock)

·   	Enclosure or casing ( CAD files can be found in github: [link])

2.2  Wiring and Connectors:

·   	Long Jumper Wires (50cm)

·   	Short Jumper wires (10cm) – for connecting components to Nano Deck

·   	Thick wires (where to use?)

·   	Screws (size) (and holders?)

2.3  Tools Needed:

·   	Soldering Iron and Solder

·   	Wire Cutter & Stripper

·   	Tweezers

·   	Screwdriver

·   	Electrical Tape or Heat Shrink Tubing

Change battery holder from parallel to series to power servo

Cut and remove connector towards wire side with pliers

Reattach red wire (if it comes out - solder if needed also)

Reattach black to other side

Wire cutter

4 long wires- buttons

4 long wires - screen

SCL/SCK

Cut open all the GND and red wires ends

4 short for RTC

LED can only handle 3.3 V - put resistor to reduce 5V

- twist resistor with led longer end (+)

- 2 long wires

Cut off servo pin ends

Both switch - fat wire - 1

WiFi one side to junction, 1 side to board

Power - 1 side to + cluster, vin

When uploading code ensure WiFi module switch is off

Insulating tape for clusters

3. Step-by-Step Assembly

### 3.1 Preliminary Steps

Gather All Parts and Prepare your workspace

- Gather every component listed in the Materials List

- Confirm you have sufficient wire lengths

- Organize tools

- Ensure good ventilation if soldering.

- Have the wiring diagram nearby for reference.

### 3.2 Mechanical Mounting & Pre-Wiring

Note: This portion may involve temporarily placing components in the enclosure or on a mounting plate. You will do the final enclosure assembly in a later step, after testing major connections.

1. 	Mount the Nano and Screw Terminal Adapter

- Align the Arduino Nano onto its screw-terminal adapter (“Nano Dock”).

- Carefully press it down, matching pin labels (D13, D12, etc.).

2. 	Position the RTC Module

- Place the RTC module securely within the enclosure in a way that allows easy access to the SDA and SCL connections.

- Ensure it does not obstruct other components.

3. 	Mount the Servo and Attach the Servo Arm

- Insert the SG90 servo into the designated servo slot in the enclosure.

- Ensure the servo shaft aligns with the feeder mechanism (gear/disc) you will attach later.

- Attach the servo arm:

  - Use a small screw to secure the servo arm onto the servo shaft.

  - Do not over-tighten the screw, as this can restrict the servo’s movement and cause incorrect angle calibration.

  - The arm should be firmly attached but still allow smooth, full-range motion when the servo operates.

- Once attached, check that the servo arm is in a neutral position before finalizing assembly.

4. 	Secure the Buttons

- Insert the buttons (B1, B2, B3, B4) into their designated button mounts [diagram] in the enclosure.

- Push them through until they fit securely.

5. 	Mount the OLED Screen

- Place the OLED (SSD1306) screen into its designated screen mount in the enclosure [diagram]

- Align the mounting holes properly so the screen sits flush.

- Ensure the SDA, SCL, VCC, and GND pins are accessible for wiring.

6. 	Place the Battery Holder

- Position the battery holder within the enclosure where:

  - It does not obstruct other components.

  - The positive (+) and negative (–) wires can easily route toward the switch and ground connections.

- Optionally, secure it with adhesive tape if necessary, to prevent shifting.

### 3.3 General Wiring Guidelines

- Cut and Strip Wires as Needed: Use a wire cutter/stripper to remove approximately 5 mm of insulation from the ends of wires before inserting them into the screw terminal adapter, soldering, or making other connections.

- Choosing Wire Lengths:

  - Long wires are required for components that are positioned far from the Arduino, such as the buttons, OLED screen, LED, servo and switches.

  - Short wires are preferable for components located close to the Arduino, such as the RTC module, WiFi module, and power switch.

  - Always route wires inside the enclosure before cutting to ensure correct lengths and prevent excess slack or strain.

- Group Connections for Efficiency:

  - Plan to group all live (5V) wires together and all ground (GND) wires together for cleaner assembly.

  - Use cable ties or wire routing clips to keep the wiring organized and prevent tangling inside the enclosure, before finalizing the connections.

  - These grouped wires will later be soldered or secured to prevent loose connections.

### 3.4 Detailed Electrical Connections

All references to “Arduino pins” assume your screw terminal adapter is labeled accordingly (D13, D12, A0, A1, etc.). If using raw Nano pins, align carefully.

## 1. Battery Holder to Power Switch and Arduino (Short Wires)

- Connect the positive (red) wire from the battery holder to one terminal of the power switch.

- Connect the other terminal of the power switch to VIN on the Arduino screw terminal adapter.

- Connect the negative (black) wire from the battery holder directly to GND on the Arduino holder.

## 2. WiFi Switch to WiFi Module (ESP-01)

- Connect a long red wire from the 3.3V pin on the Arduino to one terminal of the WiFi switch.

- Connect the other terminal of the WiFi switch to VCC on the WiFi module (ESP-01).

- Connect the WiFi module's GND pin to GND on the Arduino (use a short wire for this connection).

Wire Length:

- Long for the switch (front panel to WiFi module).

- Short for the WiFi module GND connection.

Purpose:

- Allows enabling/disabling of the WiFi module to save power when WiFi is not needed.

## 3. Servo Wiring (Long Wires)

- Connect the brown wire (GND) from the servo to the common ground (GND cluster on screw adapter).

- Connect the red wire (5V) from the servo to Arduino 5V pin.

- Connect the orange/yellow signal wire to Arduino D12 (Servo #1).

If using a dual-channel feeder, connect the second servo’s signal wire to A0 (or another free digital pin).

## 4. RTC Module (SDA, SCL) (Short Wires)

- Connect RTC SDA to Arduino A4 (SDA).

- Connect RTC SCL to Arduino A5 (SCL).

- Connect RTC VCC to Arduino 5V.

- Connect RTC GND to Arduino GND.

## 5. OLED Screen (Long Wires)

- Connect SDA to RTC SDA → Arduino A4 (SDA) (I2C bus allows daisy-chaining).

- Connect SCL to RTC SCL → Arduino A5 (SCL).

- Connect VCC to 5V.

- Connect GND to common GND.

## 6. WiFi Module (ESP-01) (Optional Future Upgrade) (Short Wires)

- Connect pin 1 to Arduino RX.

- Connect pin 8 to Arduino TX.

- Connect GND to Arduino GND.

- Connect VCC to the WiFi Switch output.

## 7. Buttons (B1–B4) (Long Wires)

- Connect one leg of each button (B1–B4) to Arduino input pins (D7, D8, D9, D10).

- Connect the other leg to GND.

Configuration:

- Each button is connected to a dedicated digital input on the Arduino.

- The Arduino code should be set to use internal pull-ups or include external resistors as needed.

## 8. LED and Resistor (Long Wires)

- Connect the long leg (anode) of the LED to a 200-ohm resistor.

- Connect the resistor’s other end to Arduino D11 or A1.

- Connect the short leg (cathode) of the LED to GND.

# 3.4 Soldering and Securing Connections

This step finalizes the electrical connections by ensuring all wires are securely attached, insulated, and neatly routed inside the enclosure.

#### 1. Soldering Critical Connections

Before soldering, twist wires together to ensure strong and reliable joints.

Solder all permanent connections, including:

- Servo wires if extended.

- Power connections (battery holder to switch, switch to Arduino VIN).

- WiFi switch to WiFi module VCC.

- LED and resistor connection.

- All grouped live (5V) and ground (GND) wires.

#### 2. Insulating and Protecting Connections

- Electrical tape is the preferred choice for insulating exposed connections.

- Apply electrical tape to:

  - Soldered connections that cannot be covered with heat shrink tubing.

  - Any exposed wire joints that need temporary insulation before enclosure assembly.

- Heat shrink tubing can be used as an alternative where applicable, such as:

  - Individual soldered joints before final securing.

  - Areas where a more durable and permanent seal is required.

#### 3. Grouping and Soldering Live (5V) and Ground (GND) Wires

- All live (5V) and ground (GND) groups must be soldered together to ensure stable power distribution.

- Bundle all 5V wires together and all GND wires together before soldering.

- Cover grouped soldered connections with electrical tape or heat shrink tubing for insulation.

#### 4. Securing Wires and Components Inside the Enclosure

- Use adhesive mounts, cable ties, or wire clips to secure wires and prevent movement inside the enclosure.

- Ensure the Arduino, RTC module, WiFi module, and battery holder are positioned securely without obstructing moving parts.

#### 5. Final Inspection Before Enclosure Assembly

- Verify that all wires are properly routed and have no tension or obstruction.

- Confirm there are no exposed wires that could cause short circuits.

- Check that all soldered joints are strong and insulated with electrical tape or heat shrink tubing.

- Ensure switches, buttons, and LED wires are positioned correctly for user access.

Once verified, proceed with closing the enclosure and securing all components.

### 3.5 Inserting the Assembly Into the CAD Enclosure

### 3.6 Final Checks and Power-Up

### 3.7 Uploading and Configuring the Code test components with example scripts