Explanation of Scripts and Changes

01_init_wifi_test and 02_wifi_test

- Scripts made to test how the NodeMCUs can connect to school wifi

- Ran into several issues which led me to create another wifi source from the Raspberry Pi instead

03_wifi_and_servo

- Scketch testing connection to Raspberry Pi wifi and testing Servo synchronisation with NTP servers (retrieving time)

04_LED_before_food

- Script simulating the feeding event (LED on for 5 seconds, 3 seconds in, servo moves to give feed)

- Testing how this event can be synchronised to given times of the day

tirtha_sketch_1

- Test script for new prototype designed by tirtha made of Arduino board, RTC time module, buttons and an LCD screen

06_15dec24_sketch

- Test script for new prototype designed by tirtha with added functionality to buttons

  - Button 1: Feed at chosen time

  - Button 2: Immediate test of servo and LED

  - Button 3: Return to default mode - feeding at set times fo the day

  - Button 4: Display menu of button functions on the screen

- Due to delays related to component power consumption - the countdowns (e.g. countdown to feeding event) was not accurate - countdown was much longer than 1 seconds

05_with_sleep_mode

- Testing added sleep mode functionality to try and improve power usage of the system

  - Sleep when buttons have not been pressed in the last 15 seconds

08_single_feeder_16dec

- Sleep mode removed — will be using less power consuming and smaller screen

- Also improved font size, screen placement etc